MATERIAL SAFETY DATA SHEET

PRODUCT : CATHOLYTE

PRODUCED AT pH 11.5 - 12.

###### Note that this only relates to CATHOLYTE produced by ECT Ltd. equipment

###### at the quoted salt concentration

# SECTION I. PRODUCT IDENTITY

Chemical Name: 				Catholyte

Common name:				Electrochemically activated water containing 0.3% NaCl

Product Type:				Aqueous dilute mixed reductants

| Composition |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ingredient |  | CAS-No |  | EINECS-NO |  | Wt/Vol % |  |  |
| Water |  |  |  |  |  | 99.7% |  |  |
| Sodium chloride |  | 7647-14-5 |  | 231-598-3 |  | 0.3% |  |  |
| After activation |  |  |  |  |  |  |  |  |
| OH¯/OH۠• |  |  |  |  |  | < 170ppm (0.0170 %) |  |  |
| Na+ as NaOH |  |  |  |  |  | <230ppm (0.023%) |  |  |
| Na+ as NaCl |  |  |  |  |  | <630ppm (0.063%) |  |  |
| H2 |  |  |  |  |  | 4ml/L |  |  |
| HO2¯ |  |  |  |  |  | <3ppm (0.0003%) |  |  |

The mixed reductants are in dynamic equilibrium initially and gradually revert to the alkalised initial ingredients after time. Figures shown are maximal.

ECT Ltd Product Name:		CATHOLYTE

# SECTION 2. PHYSICAL /CHEMICAL CHARACTERISTICS

Appearance:					Homogeneous clear, liquid.

Boiling point:					1000C

Sp Gravity:					1.02 – 1.06g/ml

Odour:						No odour

Taste:						Mild saline/soapy

Chemical: 					pH 11.5 - 12

Oxidation Reduction Potential: 		ORP = -900±50mV

Solubility:					As for water

# SECTION 3. FIRE HAZARD AND CONTROL

Flammability:					Not applicable

Flash point (°C):				Not applicable

Extinguishing media:				Not applicable

Special fire-fighting procedures:		Not applicable

# SECTION 4. REACTIVITY DATA

Stability:	CATHOLYTE is an aqueous solution containing metastable reductants which lose activity immediately on encountering reactants or during approximately 24 hours on storage.

No hazardous reactions known when used for its intended purposes

Incompatibility (material to avoid):	CATHOLYTE is reactive with acid solutions as per water.

Hazardous decomposition or by-products:	CATHOLYTE deactivates to the quality of its starting source water plus its added Sodium Chloride of 3000ppm plus Sodium Hydroxide of 400ppm formed in process.

PRODUCT:  AVERSOL®

# SECTION 5. ENVIRONMENTAL CHARACTERISTICS

Degradability:	Shelf life up to 24 hours if stored between 5 and 35’C

CATHOLYTE deactivates to the quality of its starting source water plus its added Sodium Chloride of 3000 ppm and Sodium Hydroxide 400ppm formed in process.

Hazards:	CATHOLYTE, generated at pH ≤12.0 is non-hazardous to human and animal tissue.

Other effects:					Not applicable

# SECTION 6. HEALTH HAZARD DATA

Acute oral toxicity:				None observed

Acute dermal irritation:		 	Non-irritating when concentrated CATHOLYTE is applied

Acute eye irritation:  				Non-irritating when concentrated CATHOLYTE is applied

Inhalation: 					Non-irritating when concentrated CATHOLYTE is applied Occupational exposure limits:			None

Health hazards:                                   		There are no known health hazards.

# SECTION 7. EMERGENCY AND FIRST AID PROCEDURES

Signs and symptoms of poisoning:		Not applicable

First Aid procedures:				None specified

Skin contact:					No reports of adverse skin reactions after exposure

Eye contact:					No reports of adverse ocular reactions after exposure

Ingestion: 					No reports of adverse reactions after ingestion

Inhalation: 					No reports of adverse reactions after inhalation

Emergency antidote:				None

# SECTION 8. MEDICAL ADVICE

AVERSOL has been extensively tested in both humans and animals, and poses no known threat to the welfare of either.

# SECTION 9. PRECAUTIONS FOR SAFE HANDLING AND USE

Handling concentrated product: 		None

Handling or applying diluted product:	None

Leaks and Spills:	Leaks and spills can be removed as for ordinary water.

Waste disposal:	CATHOLYTE can be disposed of in municipal drains without adverse effects after use. Local environmental regulatory requirements should be followed, which may require CATHOLYTE to be deactivated. This can be done by mixing with ANOLYTE.

Storage: 	Not applicable

Other precautions:	None

FOR FURTHER INFORMATION REFER TO ECTG LTD.

DISCLAIMER: 	This information is based on our current knowledge and is intended to describe the product for the purposes of health and safety requirements only. It should not, therefore, in itself be construed as a guarantee of any specific quality relating to the product.

DATE PREPARED: 	December 2006

ECT Ltd 							Distributed by:

340 E. Warm Springs Road, Suite 1A			Zap Water Technology, Inc

Las Vegas, NEVADA 89119					PO Box 23205

Tel (702) 260-4100						Richfield, MN 55423

(612) 978-8435

zapwatertech@hotmail.com