MATERIAL SAFETY DATA SHEET

PRODUCT: NEUTRAL ANOLYTE (ECASOL)

PRODUCED AT STRICTLY CONTROLLED pH 7.0

###### Note that this only relates to ECASOL produced by STEL equipment at the quoted salt concentration

# SECTION I. PRODUCT IDENTITY

Chemical Name: 				Neutral Anolyte (Hypochlorous Acid)

Common name:				Electrochemically activated water containing 0.3% NaCl

Product Type:				Aqueous dilute mixed oxidants

| Composition |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ingredient |  | CAS-No |  | EINECS-NO |  | Wt/Vol % |  |  |
| Water |  |  |  |  |  | 99.7% |  |  |
| Sodium chloride |  | 7647-14-5 |  | 231-598-3 |  | 0.3% |  |  |
| After activation |  |  |  |  |  | 200 ppm (0.02%) oxidants |  |  |
| Hypochlorous acid |  | 7790-92-3 |  | 232-232-5 |  | <158.2 ppm (0.01582%) |  |  |
| Hypochlorite ion |  | 7681-52-9 |  | 231-668-3 |  | <41.8 ppm (0.00418%) |  |  |
| Ozone |  | 10028-15-6 |  | 233-069-2 |  | <1.4 ppm (0.00014%) |  |  |
| Chlorine dioxide |  | 10049-04-4 |  | 233-162-8 |  | <2.5 ppm (0.00025%) |  |  |
| Chloric acid |  | 7790-93-4 |  | 232-233-0 |  | <1.5 ppm (0.00015%) |  |  |
| Chlorous acid |  |  |  |  |  | <3.0 ppm (0.0003%) |  |  |

Chemical aspects: The short-lived mixed oxidants which are formed initially are in dynamic equilibrium and gradually revert to the initial ingredients after time. Figures shown are maximal.

Physical aspects:  The formed substances are additionally in an electrochemically energised state which relaxes gradually over a period of 24-48 hours. The ECASOL initially contains energised microbubbles formed at the electrode interface. These also dissipate over a period of 4 - 6 hours.

ECT Ltd Product Name:		ECASOL/Anolyte

TYPICAL APPLICATION: Used as fast-acting sanitizer and disinfectant.

# SECTION 2. PHYSICAL /CHEMICAL CHARACTERISTICS

Appearance:					Homogeneous clear, liquid.

Boiling point:					1000C

Sp Gravity:					1.02 – 1.06g/ml

Odour:						Mild ozonous/chlorinous odour

Taste:						Mild saline

Chemical: 					pH =7.0 ±0.5

Oxidation Reduction Potential: 		ORP = +850 mV ±100mV

Solubility:					As for water

# SECTION 3. FIRE HAZARD AND CONTROL

Flammability:					Not applicable

Flash point (°C):				Not applicable

Extinguishing media:				Not applicable

Special fire-fighting procedures:		Not applicable

# SECTION 4. REACTIVITY DATA

Stability:	ECASOL is an aqueous solution containing three levels of activity:- (1) chemically metastable oxidants which lose activity immediately on encountering reactants or  after approximately 48 hours on storage. (2) Electrochemical energy which relaxes over 24-48 hours (3) Energised microbubbles which dissipate over 8 hours.

No hazardous reactions are known when used for its intended purposes

Incompatibility (material to avoid):	ECASOL is reactive with acid and alkaline solutions as per water and likewise with hydrogen peroxide and ammonia.

Hazardous decomposition or by-products:	ECASOL deactivates to the quality of its starting source water plus its added Sodium Chloride of 3000 ppm.

# SECTION 5. ENVIRONMENTAL CHARACTERISTICS

Degradability:	Shelf life up to 48 hours if stored between 5 and 35’C

Neutral ECASOL deactivates to the quality of its starting source water plus its added Sodium Chloride of 3000 ppm

Hazards:	ECASOL, generated at pH 7.0 and 200ppm FAC is non-hazardous to human and animal tissue.

Other effects:					Not applicable

# SECTION 6. HEALTH HAZARD DATA

Acute oral toxicity:				None observed

Acute dermal irritation:		 	Non-irritating when full strength ECASOL is applied

Acute eye irritation:  				Non-irritating when full strength ECASOL is applied

Inhalation: 					Non-irritating when full strength ECASOL is applied Occupational exposure limits:			None

Health hazards:                                   		There are no known health hazards from normal exposure

# SECTION 7. EMERGENCY AND FIRST AID PROCEDURES

Signs and symptoms of poisoning:		Not applicable

First Aid procedures:				None specified

Skin contact:					No reports of adverse skin reactions after exposure

Eye contact:					No reports of adverse ocular reactions after exposure

Ingestion: 					No reports of adverse reactions after ingestion

Inhalation: 					No reports of adverse reactions after inhalation

Emergency antidote:				None

# SECTION 8. MEDICAL ADVICE

Neutral ECASOL has been extensively tested in both humans and animals, and poses no threat to the welfare of the operator or test animal.

# SECTION 9. PRECAUTIONS FOR SAFE HANDLING AND USE

Handling concentrated product: 		None

Handling or applying diluted product	:	None

Leaks and Spills:	Leaks and spills can be removed as for ordinary water.

Waste disposal:	ECASOL can be disposed of in municipal drains without adverse effects after use. Local environmental regulatory requirements should be followed, which may require ECASOL to be deactivated or neutralized by sodium sulphite, sodium thiosulphate or mixed with CATHOLYTE if still active.

Storage: 	Optimal efficacy of the product will be prolonged if stored away from direct sunlight and in sealed opaque or tinted containers and avoiding high temperatures.

Other precautions:	None

FOR FURTHER INFORMATION REFER TO ECTG LTD.

DISCLAIMER: 	This information is based on our current knowledge and is intended to describe the product for the purposes of health and safety requirements only. It should not, therefore, in itself be construed as a guarantee of any specific quality relating to the product.

ECT Ltd 							Distributed by:

340 E. Warm Springs Road, 					Zap Water Technology, Inc

Las Vegas, NV 89119 					PO Box 23205

Tel: (702) 260-4100						Richfield, MN 55423

(612) 978-8435

zapwatertech@hotmail.com